ELECTRIC FIELD

PRODUCTION OF CONTINUOUS CHARGES

Continuous charges are also called current (or moving electrons). They are produce by a device called ‘a cell’.  There are two types of cells. Namely:

- Primary cells

- Secondary cell.

- Primary cells: These are cells that cannot be recharged once they run down. E.g., torch batteries, calculator batteries, leclanche cell, Daniel cell.

The components of the cell are shown below:

Bulb

Copper plate                                                   zinc plate

(Anode)                                                   (cathode)

Diluted H2SO4

However, primary cells have two defects. They are:

a) Polarization: As the cell works, hydrogen bubbles are released. These bubbles insulate the positive electrode and then slow it down. It could be reduced by brushing the plate or by using a depolarizer called manganese (iv) oxide to oxidize the hydrogen bubbles to form water.

b) Local Action: This defect results from not using a pure zinc plate which makes it to wear away. It is reduced by amalgamating the plate usingH2SO4.

- Secondary cells: These are cells that could be recharged when they run down using a suitable battery charger. The two major types are lead acid accumulator and the nickel cadmium cell. E.g. car batteries, motorcycle batteries, cell phone batteries and all other rechargeable batteries.

ELECTRIC CIRCUIT

SERIES AND PARALLEL ARRANGEMENT OF CELLS AND RESISTORS

a) Cell Arrangement:

i. Cells in Series: The total EMF is the sum of all the EMFS.

E1      E2     E3

ET=E1+E2+E3

ii. Cells in Parallel:

E1

E2

E3

ET=E1 or E2 or E3, if E1=E2=E3, i.e, all emfs are equal.

ET=E3, emf with the highest value, ifE3>E2>E1

b) Resistors Arrangement:

i. Resistors in Series:                R1               R2               R3

V1               V2                 V3

VT

VT=V1+V2+V3      (Circuital components in series have the same I but different V)

But V=IR

∴IRT=IR1+IR2+IR3

IRT=I(R1+R2+R3)

Dividing through by I, we have RT=R1+R2+R3

ii. Resistors in Parallel:

I1

R1

I2

IT

R2

I3

R3

V

IT=I1+I2+I3

But I=VR

∴VRT=VR1+VR2+VR3

∴V1RT=V1R1+1R2+1R3

Dividing through by V, we have:

1RT=1R1+1R2+1R3=R1R2+R1R3+R2R3R1R2R3

EXAMPLES

1. If four 3𝛀 resistors are connected 	a) In series

b) In parallel, find their net resistance in each case.

Solution:

a) In series, RT=R1+R2+R3+R4=3+3+3+3=12Ω

b) In parallel, 1RT=1R1+1R2+1R3+1R4=(13+13+13+13)=43

RT1=34

RT=0.75Ω

2. Three cells of value 2V, 2V and 3V are connected a) in series b) in parallel. Find their total emf in each case.

Solution:

a) In series, 
ET=E1+E2+E3=2+2+3=7V

b) In parallel,

Since 3V has the highest emf among the three cells, ET=3V

MULTIMETERS

- Shunt: A shunt is a low resistance usually used to convert a galvanometer to an ammeter. It is usually connected in parallel with the galvanometer.

Example:

A galvanometer gives a full scale deflection when a current of 10mA flows through it. How would you convert it to an ammeter capable of reading 3A if the resistance of the galvanometer is 5Ω?

Solution:

To do this, we connect a resistor of low resistance called shunt in parallel with the galvanometer. The value of such shunt is calculated below:

10mA = 10/1000=0.01A

G

0.01A,      5Ω

3A

2.99A

Shunt(R)

p.d across galvanometer = p.d across shunt

Igrg=IsRs

Rs=IgrgIs=0.01×52.99=0.0167Ω

Thus, a shunt of 0.0167Ω is needed.

- Multiplier: A multiplier is a high resistance usually used to convert a galvanometer to a voltmeter. It is usually connected in series with the galvanometer.

Example:

To convert a galvanometer to a voltmeter, we need a multiplier R so that the galvanometer could read 20V at full scale deflection of 10mA. Find the value of R if the internal resistance of the galvanometer is 6 Ω.

Solution:

G

R (multiplier)

0.01A

r = 6 Ω

V1               V2

20V

20V = V1 + V2

∴20=0.01r+0.0R

R=1994Ω

RESISTIVITY AND CONDUCTIVITY

Experiment shows that the resistance of a metallic conductor is directly proportional to the length of the wire and inversely proportional to the cross sectional area.

i.e, R ∝LA

∴R=ρLA

Hence, ρ=RAL

Where ρ the constant of the equation called the resistivity, expressed in ohm-meter (Ωm)

Resistivity of a wire is defined as the product of the wire’s resistance and its cross sectional area, divided by the length of that wire. It can also be defined as the resistance of unit length of material of unit cross sectional area. It is the ability of a substance to restrict heat.

If a wire has a diameter d, then  A=πr2=πd24

∴R=4ρLπd2    and ρ=πRd24L

Conductivity on the other hand, is a measurement of the ability of a substance to conduct heat. It is the reciprocal of resistivity and denoted by σ

Hence, σ=1ρ

σ=LRA=4LπRd2

Example:

A 2m resistance wire, area of cross-section 0.50mm2, has a resistance of 2.20Ω. Calculate

a) The resistivity of the wire.

b) The length of a wire which, when connected in parallel with the 2m wire will give a resistance of 2.0Ω.

Solution:

(a)	ρ=RAL

ρ=2.2×0.5×10-62

ρ=5.5×10-7Ωm

THE WHEATSTONE BRIDGE

The Wheatstone bridge is one of the most accurate method of measuring resistance. It consists of four resistors of resistance R1, R2, R3 and R4 connected as shown below.

I1R1                                       I1R2

I2R3                   I2 R4

e.m.f

G

I1R1

When no current flows through the galvanometer, the bridge is said to be balanced.

Therefore, p.d across R1   =   p.d across R3

And           p.d across R2   =   p.d   across R4

Hence, I1R1 =  I2R3   -----(1)

And      I1R2  =  I2R4   -----(2)

Diving (1) by (2), we have

R1R2=R3R4

Where R1 is the unknown resistance, R2 is a fixed resistance of known value, R3 & R4 are variable resistances of known values.

THE METRE BRIDGE

It consists of a straight uniform resistance wire AB of length 1m stretched along a metre rule. The unknown resistor X is placed at the left side, while the known R is placed at the right side.

R

X

Jockey

l1

l2

A

B

Key                         battery

When the circuit is closed, a point is located along the resistance wire with the jockey when G reads zero.

At this point, X ∞ l1 and R ∞ l2

∴X=kl1 and R= kl2

Since k is the same, ∴Xl1=Rl2

∴X=Rl1l2

Precautions:

- The value of the known resistance R should be chosen so that the balance point comes between 30cm and 70cm.

- The battery key should always be depressed before the G contact is made on the bridge wire.

- A very small current should be passed through the galvanometer to avoid damage.

- In other not to destroy the uniformity of the wire, the jockey should touch the wire lightly.

THE POTENTIOMETER

A potentiometer consists of uniform resistance wire AB of length 100cm through which a source of emf maintains a steady current.

Since the wire is uniform and current constant,   V ∝ R ∝ l

V=kL

A

B

When a potentiometer is used to compare the emf of two cells, at balance point when G reads zero,

E1E2=l1l2

MERITS OF POTENTIOMETER OVER VOLTMETER

- The potentiometer is a more accurate device than the voltmeter.

- There is no zero scale error as that associated with pointer instrument like voltmeter.

- It passes no current at the time current is being taken.

ELECTRIC FORCE BETWEEN TWO POINTS CHARGES

(COULOMB’S LAW OF ELECTROSTATIC FORCE)

Like charges repel and unlike charges attract.

Based on this; Coulomb’s law of electrostatic force came into being. The law states that, ‘The electrostatic force of attraction or repulsion FC between two charges is directly proportional to the product of the charges and inversely proportional to the square of their distance of separation.

Mathematically,

FC∝Q1Q2r2

FC=kQ1Q2r2

Where k=14πϵ0

Hence, FC=Q1Q24πϵ0r2

Where ϵ0=8.85×10-12F/m

andk=9×109Nm2/C2

ELECTRIC FIELD INTENSITY OR STRENGTH ∈

This is defined as the electric force experienced per unit charge. It is a vector quantity and expressed in N/C.

Mathematically,

ϵ=FCQ

∴∈ =Q4πϵ0r2

ELECTRIC POTENTIAL VE

The electric potential VE at a point in an electric field is defined as the work done in taking a unit charge from infinity to that point.  It is a scalar quantity and expressed in volt.

Mathematically,

VE=∈r

VE=Q4πϵ0r

WORKED EXAMPLES

Example 1. Calculate the electrostatic force that exist between two charges of values 5.3μCand-3.2μC placed at 6.0×10-4m. (k=9×109Nm2/C2)

Solution:

FC=kQ1Q2r2=9×109×5.3×10-6×3.2×10-66.0×10-42=4.24×105N

Example 2. Determine the electric potential due to charge 5.3μC placed at that point.

Solution:

VE=Q4πϵ0r=5.3×10-6×9×1096×10-4=79,500KV

Example 3: Three charges +10C,  - 20C  and 16C are distributed as shown in the figure below. Find the resultant force acting on the 10C charge.

+ 10C                                                - 20C

3m

2m

-16C

Solution

Force of attraction between +10C and – 20C is

F1=kQ1Q2r2

=  9 x 109 x 10 x 20 / 32 = 2 x 1011N (k=14πϵ0r2 9 x 109)

Force of attraction between +10C and  – 16C is

F2=kQ1Q2r2

F1

θ

F

F2

=  9 x 109 x 10 x 16 / 2 x 2 = 3.6 x 1011N(3.6 x 1011)2

Net force = √(F12+F22)

(2 x 1011)2 + (3.6 x 1011)2

= 4.2 x 1011N

Tanθ = F1 / F2 = 2 x 1011/ 3.6 x 1011  =  0.555

θ = tan – 1 0.555 = 32.280

The net force is 4.12 x 1011N in a direction 32.280 to the line joining the +10C and – 16C charges.

Example 4: two similar but opposite point charges – q  and + q each of magnitude 5.0 x 10 – 8 C are separated by a distance of 8.0cm in a vacuum as shown below.

-q                                                                            p                                                        +q

5cm                                                            3cm

Calculate the magnitude and direction of the resultant electric field intensity E at the point p. Draw lines of force due to this system of charges. (k=14πϵ0r2 9 x 109 Nm2C – 2 )

Solution

At p the direction of the field vector is towards the left since the positive charge at p is attracted towards the –q charge.

E – = -q4πϵ0r2 = 5.0×10-8×9×109(0.05)-2= 1.8 x 105NC – 1

The field vector due to +q charge is also directed towards the left since the positive charge at p is repelled by +q

E – = +q4πϵ0r2 = 5.0×10-8×9×109(0.03)-2= 5 x 105NC – 1

Resultant electric field is E = E – + E +

E = (1.8 x 105 + 5.0 x 105) NC – 1

The field is directed towards the left or towards –q charge.