Elastic Properties of Solids

Elasticity is the property of a material that enables it to regain its original shape and size after the deforming force is removed. Materials that exhibit this behavior are called elastic materials. Examples include steel, rubber, and springs.

Hooke’s Law

Hooke’s Law states that, within the elastic limit, the extension of an elastic material is directly proportional to the applied force.

Mathematically: F = k.e

Where: F = Applied force, e = Extension produced, k = Force constant or spring constant (measured in N/m)

The spring constant (k) is a measure of the stiffness of the material.

Hooke’s Law Experiment

Aim: To verify Hooke’s law using a helical spring.

Apparatus: A spring, meter rule, weights, retort stand, pointer.

Procedure

- Suspend a spring vertically and measure its initial length.

- Add known weights one at a time and record the corresponding extensions.

- Plot a graph of force (weight) against extension.

It is observed that the graph is a straight line, indicating that the extension is proportional to the applied force up to the elastic limit.

Young’s Modulus (Y) Young’s modulus is a measure of the stiffness of a solid material. It is defined as the ratio of stress to strain within the elastic limit. Y = Stress \ Strain

Work done in springs and Elastic Strings

The work done in stretching a spring or elastic string is equal to the elastic potential energy stored in it. W = ½ke^2 Where: = Work done (J), k = Spring constant (N/m), e = Extension (m) Applications of Elasticity

- Construction: Steel is used in buildings and bridges due to its high Young’s modulus.

- Springs: Found in watches, weighing machines, and suspension systems.

- Medical Use: Elastic materials are used in surgical instruments and orthopedic devices.

- Sports Equipment: Tennis rackets, fishing rods, and diving boards rely on elastic properties.

- Engineering: Used in designing shock absorbers and pressure vessels.