Electric field

Electric current, potential difference, Electromotive force

Electric current (l): is defined as the rate of flow of electric charge along a conductor.

I=Qt

Q is the quantity of charge measured in Coulomb, t is the time in second. I is the current in Ampere (A).There are submultiples of Ampere

1mA = 10-3A

1μA = 10-6A

Electric circuit

An electric circuit is the path provided for the flow of electric current.

An electric circuit consists of the source of electricity, the key or switch and the connecting wires, ammeter to measure the current, voltmeter to measure the potential difference, Resistor or load and a rheostat to adjust the flow of current.

Ammeter

Resistor

Battery/cell

key

CIRCUIT DIAGRAM

V

Rheostat

A

Closed Circuit: It is a circuit in which there is no gap (key closed) along the conducting path.

Load

Key closed

cell

Open circuit: It is a circuit in which there is a gap (key open) along the conducting path.

Load

key

Dry cell

Short Circuit: A short circuit is a closed circuit without a load. The terminals of the cell are connected together.

Key closed

cell

Potential difference ( V): The potential difference between any two points in an electric field is defined as the work done in moving a positive charge of 1 coulomb from one point in the electric field to another. Potential difference is measured in volts.

Voltmeter is used to measure potential difference.

V=workcharge

Electromotive force: Electromotive force is defined as the total work done in driving one coulomb of electricity round a circuit or the total energy per coulomb obtained from a cell or battery.

E.m.f=workcharge

Resistance: This can be defined as the opposition to the flow of charges (electrons) or current. Its unit is Ohm. It is measured using Ohmmeter.

Types of Resistors:

i. Fixed/standard resistor: They have fixed resistance. The electrical symbol is

OR

ii. Variable resistor: They are those resistors whose resistance can be varied such as Resistance box and Rheostat. The electrical symbols are

SOURCES OF ELECTRIC CURRENT

Electric current can be generated from the following sources.

1.  Chemical energy: Electrical cells store chemical energy. There are two types of electrical cell. The primary cell and the secondary cell. The primary cell cannot be recharged while the secondary cell can be recharged.

2.  Heat Energy: Electricity can be generated by thermoelectric effect using a thermocouple, which consists of two different metallic wires joined and dipped in hot water while the other end is connected to a sensitive galvanometer.

3. Mechanical Energy: Current can be obtained from the generator. The generator converts mechanical energy to electrical energy by the principle of electromagnetic induction.

4. Solar Energy: Electricity can be generated from solar energy using the solar cell. In the solar cell solar energy is converted to electrical energy.

Arrangement of resistors and Ohm’s law

Resistors can arranged in series and in parallel

When resistors are arranged in series in a circuit, the same current flows but, they have different potential differences. When they are connected in parallel, they have the same

Potential difference but different current.

OHM’S LAW

The electric current passing through a metallic conductor at constant temperature is directly proportional to the potential difference applied between its ends.

Current=Potential differencrReistance

V = IR

R is a constant of proportionality and depends on the nature of the material. The unit of resistance is ohm.

Examples

1.   A potential difference of 240V is applied to a lamp of 60 ohms resistance. What amount of current will flow in the circuit?

SOLUTION

Current=Potential differenceResistance

I=24060=4A

2.    Calculate the effective resistance in the diagram shown below.

2Ω

2Ω

4Ω

SOLUTION:

Solve the parallel first,

1RP=1R1+1R2

1RP=12+121RP=22=1, RP =1Ω

R = RP+4=1+4=5Ω

3.    A current of 3A flows in a circuit when a p.d of 24V is applied to it. The resistance across the circuit is

4Ω            2Ω

2Ω

20V

a.      Calculate the total resistance in the circuit above

b.     Calculate the current flowing in the circuit above

From question 2, Total resistance = 5Ω.

From Ohm’s law,

Current=Potential differenceEesistance

I=205=4A

Electrical energy and power

Electrical Energy: If Q coulombs of electricity flows between two points whose potential difference is V volts, then the work done (W) is given by

W = QV

JOULES = COULOMB X VOLTS

But Q = It

Then, W = VIt,  E = VIt   OR     I2Rt  OR  V2/R

Electrical Power (P):  Power is defined as the rate of doing work.

Power=Energytime

Power=VItt

Power = VI = I2R = V2/R

ELECTRIC METER READING AND BILLING

To measure or read electricity consumed in order to bill the consumer accordingly, the standard unit commonly used is the kilowatt/hour.

1 unit of the electric power is the watt

1 watt= 1 volts x 1 ampere

Hence, the electrical meter is a device used for recording the number of units of electrical energy consumed in a house over a period of time.

1KWh = 1000 x 60 x60J

= 3.6 X 106 Joules of energy

The actual cost per unit to electrical energy varies with the nature of the use to which the energy is put.

The AEDC also charges a fixed charge and meter maintenance charge every month. This also varies according to the tariff rate of the consumer.

Example: 1. the tariff rate of a consumer is N4. 00 per unit. The present meter reading is 3760 while the previous reading was 3648.

- What is the consumption of the household after one month?

- If the fixed charge is N30 and the meter maintenance charge is N100. What is the current charge of the consumer?

SOLUTION: i. the consumption of the household is the difference between the present reading and the previous reading.

- – 3648 = 112units.

ii. The energy charge of the consumer = 112 x 4. 00 = N448. 00k

Fixed charges = N30

Meter maintenance charge = N100

Current charges =N(448 + 30 + 100) = N578 . 00

FUSE AND CIRCUIT BREAKER

Fuse is a safety device containing a short length of thin conducting wire, which melts and breaks the circuit, if the current exceeds a certain safe value or maximum operating limit.

Circuit Breaker on the other hand works in the same way as the fuse. It is usually placed beside the electric meter such that the current enters it first before going into the meter.

Connecting wire

Continuity Tester: To detect faults in a circuit, we use a continuity tester. It is used to test whether a circuit is continuous or broken at a certain point.

Electric bulb

Cell