TOPIC:

Triangle and its types

A triangle is a plane figure bounded by three straight lines. The figure below shows the typical shape of a triangle. The sum total of all the angles in a triangle is 1800.  Triangles are described by

- The lengths of their sides

- The sizes of their angle; and

- A combination of both lengths of sides and sizes of angles

Types of triangle

Hypotenuse

Right angled triangle: - a right-angled triangle has one angle equal to 900. The side opposite the 900 is called the hypotenuse.

- Equilateral triangle: - An equilateral triangle has all the three sides and angles equal.

- Isosceles  triangle:  an Isosceles triangle has two equal sides and angles

- Scalene triangles: - a scalene triangle has three unequal sides and angles.

- Obtuse-angled triangle:- it has one of its angles more than 900

Inscribed and circumscribed circles to a given triangle

Inscribed circle to a given triangle

The three sides of the given triangle are tangential to the inscribed circle

The procedure for drawing the inscribed circle to a given triangle is as follows:

- Draw the given triangle ABC.

- Bisect any two angles of the triangle. The bisecting lines will intersect at O, which is the centre of the inscribed circle.

- Draw a perpendicular lines to any sides from O to meet the sides at D

- With O as centre and radius OD draw the inscribed circle.

Circumscribed circle to a given triangles

The circumference of the circumscribed circle to a given triangle touches the vertices of the given triangle.

The procedure for drawing the circumscribed circle to a given triangle is as follows:

- Draw the given triangle ABC

- Bisect any two sides of the given triangle. The bisecting lines intersect at O, which is the centre of the circumscribed circle.

- With O as centre and radius AO draw the circumscribed circle.

QUADRILATERAL

What is a Quadrilateral?

A quadrilateral is a plane figure bounded by four straight sides and whose four angles add up to 3600. The line joining two opposite corners of quadrilateral is known as diagonal.

Examples of Quadrilateral

- Squares

- Rectangle

- Parallelogram

- Rhombus

- Trapezium

- Kite etc.

Construction of A Square

E.g.: How to construct a square given that one of its sides is 60mm

Procedures

- Draw the given side AB to be 60 mm.

- With centre A and a convenient radius, draw a semicircle at A. use the semicircle to draw a vertical line at AC

- With center A and a radius AB, draw an arc to cut the vertical line at D with center B and with the same radius, draw an arc at C. Again with center D and the same radius, draw another arc to cut the one at C.

- Join point D to C and C to B. ABCD is the square required.

Construction of a Rectangle

E.g.:	Constructing a rectangle 60mm in length and 35mm in width

Procedure

- Draw line AB 60mm to be the horizontal length.

- Construct a perpendicular at B, and on the perpendicular lines, mark off 35mm at C, using your compass.

- With center A and Radius 35mm, draw an arc at D.

- With center C and radius AB (60mm),draw an arc to cut the initial arc at D

- Join C to D and D to give rectangle ABCD.

Regular and Irregular Polygons

A polygon is a plane figure bounded by three or more straight line segments e.g. quadrilateral, hexagon, heptagon, octagon, and pentagon. A polygon can either be regular or irregular.

Regular Polygon

A regular polygon is one that has all sides equal

Irregular Polygon

Irregular polygon is one that has unequal sides

Construction of hexagon using a pair of compasses

Hexagon is a figure with six sides

Procedure

- Draw a circle with radius equal to the length of the side given, and draw the horizontal diameter

- With A as center, and the same radius (30mm), cut the circle above to give AC, and below to give AD

- With B as center, and the same radius (30mm), cut the circle above to give BE, and below to give BF.

- Join AD,DF, FB ,AC, BE and CE to obtain the required hexagon

Construction of Pentagon using a pair of compasses

USING GENERAL METHODS i.e. EXTERNAL ANGLE = 3600/N

PROCEDURE

- Obtain the external angle of the required polygon by dividing 3600 by the number of sides (N), i.e. 5 for a pentagon is 3600/5 = 720

- Draw a horizontal line and mark off AB equal to the given base

- With A and B as centers, using protractor, mark off 720 as external angles at both ends; and also mark off lengths equal to AB at both A and B points, to give the required pentagon.

- With C and D as centers, using the protractor, mark off 720 again and extend the lines to intersect at E to give the required pentagon

- The figure ABDEC is the required pentagon.

PLANE FIGURES

Construction of areas of regular plane figures

The area of a plane figure is the space occupied by that figure. Area is usually the product of two lengths and its unit is given in cm2 or m2. The areas of plane figures (also known as polygons) can be obtained through the following methods:

a)	Triangles: The area of a triangle is one-half the product of base and height.

b)	Parallelogram: The area of a parallelogram is the product of one side and the perpendicular distance between that side and the opposite side.

c)	Quadrilateral: A quadrilateral is the combination of two triangles. The areas of the two component triangles are found and added to give the area of the quadrilateral

d)	Square or Rectangle: The area of a square or a rectangle is defined as the product of length and breadth

e)	Trapezium: The area of a trapezium is one-half the product of the sum of parallel sides and the distance between them

f)	Regular polygon: The area of a regular polygon is found by joining the vertices of the polygon to the centre so as to form several triangles. The area of the regular polygon is equal to the area of one of the triangles multiplied by the number of sides.

Enlargement/Reduction of plane figures

An object is enlarged when all of its dimensions are changed in the same ratio. Thus, enlargement is also called transformation. Each length of the objects is multiplied by the same number called scale factor to obtain the enlargement.

When the scale factor is less than one, the original objects is reduced.

I.e. Scale Factor = length of enlargement/corresponding length.

Examples of enlargement/reduction of plane shapes include:

ENLARGEMENT

i) Drawing a triangle which is twice the size of the original triangle

Procedure;

- Draw the given triangle ABC

- Draw DB perpendicularly to AB, such that BD=AB

- With center A, and radius AD, draw an arc to meet AB at E

- From E, draw EF parallel to BC, with F on line AC

- Draw lines to join AEF to give the triangle double in size

ii)	Drawing a square which is twice the size of the original square

Procedure;

- Draw the given square ABCD

- Draw one of the diagonals, AC.

- With A as center, and radius AC, draw an arc to meet AD at E.

- AE is one side of the square.

- With radius AE, and center A, mark G on line BA produced

- With the same radius, and center E, mark F. join G and F with a tangent to complete the square.

REDUCTION

Reducing a Plane Figure to half its area

Procedure

- Draw the given square ABCD

- Draw the diagonal AC and BD to intersect at O.

- With center C and radius CO, draw an arc which meets CD at E. CE is the length of one side

- With radius CE and center E, strike an arc above CD at F.

- With radius CE and center C, strike another arc above line CD and G.

- Join EF, FG and GC to obtain the square CEFG half the size of the original.