Production of Wood Materials

Wood is one of the oldest materials in technology. It is composed of cellulose, lignin and other minor materials such as starch, resins, wax and gum. When closely observed, wood is seen to be made up of tiny thread-like units called fibres.

Wood is classified as either hardwood or softwood.

Hard Wood: These are got from deciduous tree. They have broad leaves and their seeds are enclosed in cases. Some examples include Mahogany, Afara, Opepe, Abura, Omo, Agba, Sapele, Oak, Jarrah and Teak.

Soft Wood: These are got from coniferous trees (i.e. trees that bear naked seeds which are in cones). They have narrow leaves and grow in the temperate regions of the world. Examples include cedar, pine, fir, larch, spruce and European whitewood.

The stages of processing wood before becoming useful are:

- Felling

- Conversion

- Seasoning

- Preservation.

Trees undergo two types of growth:

- Primary or vertical growth. This takes place mainly near the tips of the root and shoots of the tree and continues for the most part of the plant’s life.

- Secondary or horizontal growth. This results in increase in girth and takes place mostly in the cambium.

THE MAIN PARTS OF A TREE.

- The Roots

- The Stem / Trunk

- The branches

- The bark

- The Leaves

The felling of trees for timber use is done broadly in two areas in Nigeria.

- The Free Area: This is the area outside the forest reserve area and it is owned by individuals. The amount of money paid to the government by the timber contractor for permission to cut down any tree in the Free Area for commercial purposes is called Tariff.

- The Forest Reserve Area: This is the government owned area. The forest reserved area is guarded by the forest guards. Hence, it is difficult for anybody to cut down any tree in the forest reserve area without the permission from the government. The amount of money paid for the timber to be cut down in the forest reserve area depends on the volume of the trees to be cut down. Hence, this is termed O.T.V i.e. Out Turn Volume.

CONVERSION OF WOOD.

The conversion of wood is the process of sawing logs of timber with wood working machines (Sawing machine) in the Sawmills into planks of required size or marketable size or commercial sizes. The popular methods of conversion of logs of timber into planks are:

- Plain Sawn Method: The plain sawn method is also known as through and through method. In plain sawn method, planks are sawn parallel to the axis of the tree trunk. This method of sawing planks is used for producing planks which are needed for doors, windows and roofs of houses. The planks are cut parallel to the axis of the wood.

- Quarter Sawn Method: This method is simply the sawing of timber plank after plank. The Quarter sawn method is the method of conversion of timber along the rays of the wood. In quarter sawn method, planks are cut at right angles to the growth rings. This method of sawing planks is used for producing planks which are cut with the growth rings running through the thickness of the planks.

- Tangential Sawn Method: Tangential sawn method of sawing planks in which the planks are cut so that their wide edges are tangential to the growth rings.

Some of the machines used for conversion of trees are:

- The circular saw

- The horizontal log band saw

- The vertical log band saw

- The frame saw.

SEASONING OF WOOD.

Seasoning of wood is the process of drying or reducing the excess water in the wood. The excess water is called moisture content.

Methods of seasoning wood:

There are two methods of seasoning wood.

- The Air Seasoning: This is also called Natural seasoning. This method of seasoning wood involves stacking the wood in the open shed for a long period of time and allowing it to dry naturally. The stacked planks (wood) are arranged on top of one another with pieces of small wood called stackers in between them in order to allow free circulation of air.

- The Kiln Seasoning: This method of seasoning wood, involves stacking the timber in a specially heated chamber. The planks are stacked in the same way as in the air seasoning, but the planks are placed in a specially heated chamber called kiln. In this process, the rate of drying planks is faster than air seasoning method.

REASONS FOR SEASONING WOOD

- It makes the wood become lighter in weight.

- It makes the wood become more stable.

- It makes the wood to take paints, polishes and preservatives easily.

- It makes the wood to be durable i.e. last longer

- It reduces the attack of fungi and insects on wood.

- It reduces the moisture content of the wood.

% Moisture content (M.C) present in a wood:

M.C      = Wet weight 	     Dry weight    X	100

Dry weight.			1

Example 1

A sample of wood weighs 70kg when wet and after seasoning weighs 40kg. What is the percentage moisture content?

Solution:

M.C = Wet sample – Dry sample      X   100

Dry sample                   1

Wet sample= 70kg

Dry sample= 40kg

M.C= ?

M.C =     70  - 40      X    100

40                  1

=  30        X   100

40                 1

M.C      =     75%

Defects in timber

A defect in timber is any feature in timber that reduces the quality and the market value of the timber.

Types of defects in timber.

THE NATURAL DEFECTS:

The Natural defects in timber are those defects that are caused by natural occurrences such as thunder, rain, growth circumstances etc. Examples of natural defects are:

Shakes: These are disruptions in the wood fibre which show up as splits and separation of parts of the stem of a tree.

Examples:

Heart shakes: This is visible on the cross-section of the centre of the log, extending along the rays

Star Shakes: This is similar to the heart shake, but with more splits like star.

Cup Shakes: These are visible cracks which go round the growth ring of the wood.

Ring Shakes: These are circular cracks on the timber.

Knot: knots are caused by branches.

THE ARTIFICIAL DEFECTS:

The artificial defects are those defects that are caused by human beings through careless handling of the wood while processing.

Examples:

- Cup:  This is the concave curvature across the face of the timber.

- Bow: This is either concave or convex curvature along the length of the plank.

- Check: This is the separation of the wood along the grains of the wood.

- Twist: This is the spiral form of distortion along the length of the plank.

- Insect Attack: This is the visible minute holes seen on the surface of the wood. The insects always attack sap wood.

- Warps: The warp in timber is called twists. They are caused by improper seasoning procedures of wood fibers.

Wood preservation.

This is the process of prolonging the life span of wood by the application of wood preservatives to prevent insect and fungal attack.

Wood Preservatives: These are the chemicals used to poison the destructive agents in the wood. E.g. fungi, insects, and termites. Some of the chemicals used as the wood preservatives are solignum, creosote, phenols, paints etc.

Types of wood preservatives:

- Water soluble preservatives: These are the chemicals that can be mixed thoroughly with water only.

- Oils soluble preservatives: These are the chemicals that can be mixed thoroughly with oils only.

- Solvent soluble preservatives: These are the chemicals that can be mixed thoroughly with evaporating liquids only.

Methods of Applying the Preservatives:

- Brushing Method: This is the process of using brush to apply preservatives to the wood.

- Spraying Method: This is the process of using spraying gun to apply preservatives to the wood.

- Cold Immersion Method: This is the method of immersing (dipping) planks in an unheated solution of preservatives for one or two days.

- Open Tank Method (Hot and Cold Method): This is the process in which the plank is immersed in a tank containing hot liquid preservatives. This process is normally used for Treating poles and fence posts.

- Pressure Treatment Method: This is process in which planks are placed inside an enclosed metal cylinder. In the process, the preservatives are being forced into the plank under high pressure. This enables the wood to be preserved either by pumping the solution into the entire cell cavity or by merely coating the cell wall with the preservatives and finally draining the cell cavity, leaving the cell saturated with preservatives, when the process is completed.

Properties of Good preservatives:

- It must be poisonous to the destructive agents.

- It must be safe to handle

- It must be permanent to the wood on application

- It must be easy to apply on the wood

- It must be chemically stable for a long time on application to the wood.

PRODUCTION OF METALS

Metallic materials are materials which are referred to as good conductors of heat and electricity. Metals are gotten from Iron ore which is the raw material for metal production. The Iron Ore is dug from the ground along with the impurities (coke, hot air, limestone with Iron Ore).

When the Iron Ore is dug up from the ground, it cannot be used as it contains some impurities, it undergoes certain processes during which these impurities are removed and other substances are added. Since these impurities cannot be removed from iron-ore in its solid state, it is converted into a liquid state by heating. Thus process is known as smelting. The equipment used for this process is called a furnace. After removing these impurities from Iron Ore, the product left is called Pig-Iron. The Pig iron in its ordinary state cannot be used without further processing; this is because pig iron contains saturated iron and carbon. This is weak and can break easily. Hence, it forms the basis of all ferrous metals containing about 45% carbon.

In order to render pig iron usable such as cast Iron, steels and wrought Iron, the amount of carbon it contains must be reduced.

- Cast Iron: This is obtained when pig Iron is put into a cupola furnace. In this furnace, the carbon content is reduced to about 3.5%. The molten Iron is then poured from the furnace into a container, called mould, so as to solidify it. This process of pouring molten iron into a mould to solidify into a described shape is called casting, hence, the name cast iron.

Examples of cast iron

- Ductile (chilled) cast iron

- Malleable cast iron

- Grey cast iron

- White cast iron.

- Steel: This is obtained when the carbon content of pig iron is reduced to 1.5 or less. Steels are named according to the carbon content present in them.  These steels are called Plain Carbon Steels because they are made up of carbon and iron.

- Low carbon steel: (mild steel) this contains about 0.3% or less of carbon.

- Medium carbon steel: This contain between, 0.3%- 0.5% of carbon.

- High carbon steel: This contains between, 0.6%- 1.5% of carbon.

Furnaces used are open hearth and Bessemer converter.

- Wrought Iron: The wrought iron is obtained by removing the carbon content present in the pig iron. It is soft, easily bent and does not break or rust easily. This is almost pure iron with little slag on it. The wrought iron has no carbon content in them. It is almost rust proof. It is malleable and ductile and can be easily forged into the required shape. Wrought Iron is used in the manufacture of chairs, boilers, plates, gears and so forth. Annealing: is the process by which metals can be softened.

Alloy steels

All steels are alloys because they contain both iron and carbon. Alloy steel is a combination of plain carbon steel and one or more other elements which are placed in the electric furnace and then heated. This molten mixture is then poured into a mould container and then cooled to solidify. The alloy steels are:

- High tensile steels: This is also known as Nickel- chrome alloys. These contain iron, carbon, nickel and chromium. They are used for making machine parts which require high tensile and strength.

- High Speed steels: These contain iron, carbon, tungsten and chromium. Other forms of high speed steels contain cobalt and vanadium. They are used as cutting edges in machines where friction causes excessive heat due to continuous cutting.

- Stainless steels: These contain Iron, carbon, Nickel or chromium. Nickel and chromium are principally used as coating for the metals so as to make it resistant to corrosion. They are produced in electric furnace.

PROCESSING OF NON-FERROUS METALS:

The non-ferrous metals are the metals which do not have iron content in them, therefore they are not magnetic.

Examples: Copper, Aluminum, Zinc, Gold and Tungsten.

Kiln furnace

A furnace is a solid structure in which a lot of heat energy is generated to produce metal. The different types of furnaces in which different types of metals are produced are:

Blast furnace: This is the structure used for producing pig-iron from Iron Ore.

Bessemer converter: This is the structure used for producing steel from pig iron.

Open hearth furnace: This is the structure used for producing steel from pig-iron.

Electric Arc-furnace: This is the structure used for producing stainless steel.

Cupola furnace: This is the structure used for producing cast iron from pig-iron.

Pudding furnace: This is the structure used for producing wrought-iron.

Reverberating furnace: This is the structure used for producing non-ferrous metals.

Metal Alloy.

An alloy is a mixture of two or more metals. A non-ferrous alloy is a mixture of two or more non-ferrous metals.

Examples of non-ferrous alloys are

Brass: a mixture of copper and zinc

Bronze: a mixture of copper, Tin and Phosphorus.

Soft solder: Lead + Tin

Duralumin: a mixture of copper, Aluminum and Magnesium

Production of non-metallic materials

Processing of ceramics:

In the processing of ceramic materials, mud and clay are obtained and cleared of all the impurities. Afterwards, the materials are mixed with water thoroughly pounded until a smooth, even mixture is obtained. The next process is shaping of the mixture into the desired objects. The art of making moist clay materials into various objects is called pottery. There are two methods by which ceramic materials can be processed.

- The Pinch method: This is the traditional method of using fingers to carefully mould and shape objects.

- The Potter’s wheel method: This is the method with a rotating disc upon which the clay is mould.

There are 4 stages involved in the production of ceramics:

- Molding

- Shaping

- Decorating.

- Firing.

In pottery the following are involved:

- Mixing

- Shaping

- Drying, and

- Firing.

Processing of Glass

The mineral glass belongs to the group of ceramics materials; it is made from inorganic resin made from refining petroleum products. Most glasses in use are the inorganic type. Basically, inorganic glass is produced by melting together various proportions of silica, lime-stone, dolomite by firing them in a furnace. The molten substance is allowed to cool slowly through the process called Annealing.

Plastics

Plastics are commonly used both as household items and in big industries. Ordinarily, it is known as a non-metallic lightweight material which comes in various colors. The manufacture of plastics involves the combination of several different components chiefly obtained from petroleum, cellulose, and coal.

Properties of Plastics

- Plastics are lightweight materials, much lighter than wood, ceramics and metals.

- They have low resistance to heat. They are easily deformed by heat.

- They do not rust or corrode.

- They are good insulators of heat and electricity and that is why they are used as handles of pots, pressing irons, plugs, etc.

Classification of Plastics.

Thermoplastics. These are plastics which become molten again under heat and can therefore be remolded. Examples include most plastic articles used in the homes such as cups, buckets, jerry cans etc.

Thermoset. They are those plastics which cannot be melted into liquid form by heating. Examples are those used in handles of pressing irons, knives and cooking pots. Thermosets are used where high temperature is anticipated.

Manufacturing and Processing of Plastics.

Plastics are derived from organic materials obtained from petroleum. Its production can simply be broken up into the following stages:

- Obtaining the basic unit called monomer from petroleum.

- Polymerization (i.e. joining a large number of molecules) of the monomer to form resins.

- Combining the resin with appropriate materials to produce the required plastics.

- Molding and shaping of the plastics into different forms.

Methods of producing plastics.

The plastic materials can be molded into any of the required form into any of the required shape when wet through any of the following processes.

- The compression molding: This is the method of forming plastic materials in a mould by means of the application of pressure and heat energy. This method is employed in forming thermoset plastics.

- The injection molding: This is a method of forming an object by injecting hot molten metals by means of plungers.

- The Extrusion: This is the method of heating a plastic material and forcing it through a mould followed by continuous cooling. It is a method used in the manufacture of thermoplastic pipes.

- The Calendaring: This is a method for producing thermoplastic sheets in paste form between heated and cooled rollers.

- The Vacuum forming: This is an enclosed method from which air had been completely removed. It is a method for making plastic cups.

Methods of Processing Rubber.

Rubber: is an elastic organic material. Rubber can easily be stretched, and it can return to its      original position when the force is released. There are two types of rubber—natural rubber and synthetic rubber.

Properties of Rubber

- They are elastic.

- They are water resistant and will float on water.

- They are good electrical insulators.

Uses of Rubber:

- Rubber is a very good shock absorber and is therefore used for making soles of shoes as well   as tyres and inner tubes for motor vehicles

- It is also used for making rubber sandals, rain boots, catapults, etc.

- For covering electric wires.