SIMPLE MACHINES

An arrangement by which work can be done conveniently on a load or against a resistance is known as machine

Simple machine is a machine in its simplest form.  They are devices that use energy to do work. The work is being done by the machine when a small effort is used to overcome a large resistance.

Simple machine can be classified into different categories namely the lever, the pulley, the inclined plane, the wedge, the wheel and axle, the screw, the hydraulic press.

Some common examples of simple machines are, scissors, drill brace, the shovel (a form of lever), the pulley at the top of a flagpole, the steering wheel of an automobile (a form of wheel and axle), and the wheelchair ramp (a form of inclined plane). An everyday example of a complex machine is the can opener, which combines a lever (the hinged handle), a wheel and axle (the turning knob), and a wedge (the sharpened cutting disk).

Mechanical advantage/Force ratio of a machine is defined as the ratio of the load to the effort.

Mechanical Advantage=LoadEffort=Le

If the laod is bigger than the effort, the mechanical Advantage is greater than one.

In pratice,all machines have some friction in them and this reduces the efficiency. Part of the work put into a machine is thus always wasted in overcoming friction and in moving some parts of the machine. Thus, no machine is hundred percent (100%) efficient.

Velocity Ratio of a machine is defined as the distance moved by the effort to the distance moved by the load.

Velocity ratio=Distance moved by EffortDistance moved by Load=xy

EFFICIENCY OF A MACHINE

The efficiency of a machine is defined as the ratio of work obtained from the machine to work put into the machine expressed in percentage.

It also defined as the ratio of work output  of the machine to the total work input expressed in percentage.

Efficiency =   Work output    X  100%

Workinput

DERIVATION OF EFFICIENCY

The level of the efficiency of a machine can be determined by the  work output and work input  of the machine or its velocity ratio and mechanical advantage.

Therefore:

Efficiency (E)   = Workoutput       x  100%

Workinput

Efficiency (E) = Work done in load       x 100%

Work done in effort

Since Work = force x distance

Efficiency (E) =     Load(L) x distance moved by load(y)    x 100%

Effort(e) x distance moved by effort(x)

Therefore E =L × ye × x

Since Velocity ratio (V.R) = xy   or  1V.R =yx  and

Mechanical advantage (M.A) = Le

Thus E =M.AV.R ×100%

Efficiency (E)  =    Mechanical Advantage      x 100%

Velocity Ratio

Example 1:

A system of lever with velocity ratio 30 overcomes resistance of 2500 Newton when an effort of 125 Newton is applied to it, calculate

- The mechanical advantage of the system.

- It’s efficiency.

SOLUTION:

Mechanical Advantage=LoadEffort=2500125=20N

Efficiency=M.  AV.  R x 100

Efficiency=2030 x 100=2003=66.75%

LEVER

In a lever the relative positions of force(F), Effort(e) and Load(L) may vary and this leads to different types of lever. The lever operates on the principle of moment.

FIRST ORDER LEVER: In first order lever, the fulcrum is between the load and the effort  e.g crowbar, claw hammer,  pliers, scissors, see-saw e.t.c

SECOND ORDER LEVER: In second order lever, the load is between the effort and the fulcrum. E.g wheelbarrow, bottle opener, nut cracker

THIRD ORDER LEVER: In third order lever, the effort is between the load and fulcrum. E.g Forearm of a human being, tongs e.t.c

PULLEYS:

A simple pulley is a fixed wheel with a rope passing round a groove in its rim. A load is attached to one end of the rope while effort is applied at the other end. If there is no friction, load is equal to the effort which is equal to the tension in the rope.

L =T =E, therefore M.A =V.R

For a block and tackle or systems of pulley of n pulleys, the velocity ratio is equal to the number of pulley.

Velocity ratio = number of pulley = n

A system of pulley is used for lifting loads. They are used by builders for hauling heavy loads to high floors or in loading and unloading ships.

CALCULATIONS:

Example 1.    The velocity ratio and efficiency of a system of pulleys are 6 and 80%, respectively. How much effort is required to lift a load of mass 120kg with this system? [g = 10ms-1]

SOLUTION:

V.R = 6, Efficiency = 80%, Load = 1200N, e = ?

Calculating M.A,

Efficiency=M.  AV.  R x 100

80=M.  A6 x 100

M.A=80 X 6100=4.8

M.A=LoadEffort

Effort=LoadM.A = 12004.8=250N

INCLINED PLANE:

An inclined plane is a sloping surface used for pulling or pushing a load up, rather than lifting them vertically.

X

h

L

E

θ

Velocity ratio=Distance moved by EffortDistance moved by Load=xh=1sinθ

HYDRAULIC PRESS:

Hydraulic press is a device used to produce a very large force to compress or lift up a heavy load. It is used in the printing press where a large force presses the type with ink on it against the paper. Pressure is transmitted equally to all parts of a liquid at the same level.

If A1 is the area of small piston and A2 area of the large piston, pressure (P) is transmitted equally at the same level, then

P = EA1,    E = P X A1

P = LA2,,    L=P X A2

Therefore a small effort lifts a large load.

M.A=LE = A2,A1,=R2r2

xy = A2,A1,=R2r2=V.R

If x and y are the distances moved by E and L respectively,  A1x = A2y

CALCULATIONS:

Screw:

Mechanical fastening device consisting essentially of an inclined plane wound spirally around a cylinder or a cone. The ridges formed by the winding planes are called threads, and depending on the intended use, the threads may be square, triangular, or rounded in cross section.

PARTS OF A SCREW THREAD

A screw thread is made of the following parts:

1.  Thread or Lead: Is the distance along the screw's axis that is covered by one complete rotation of the screw (360°).

2.   Pitch: The distance between two corresponding points on adjacent threads

3.  Plane or Start: It can be Single-start or double-start. Single-start- means that there is only one "ridge" wrapped around the cylinder of the screw's body. Double-start" -means that there are two "ridges" wrapped around the cylinder of the screw's body Each time that the screw's body rotates one turn (360°), it has advanced axially by the width of two ridges.

Another application of the screw is Screw jack , Ratchet brace.

When a screw is turned through one complete revolution by the application of an effort ( on the screw head, using the handle of the ratchet brace, or the Tommy bar of the screw jack), the load moves a distance equal to the pitch(P), which is the distance between consecutive threads.

V.R=2πRP

WHEEL AND AXLE

A wheel and axle is really two machines in one because it consists of a wheel mounted rigidly upon an axle or drum of smaller diameter, the wheel and the axle having the same axis.

The wheel & axle device is similar to the lever simple machine. The wheel acts as the lever and the axle acts as a fulcrum. The lever applies force to the fulcrum, causing something to move.

If a rope is fixed onto the wheel and wound round it, leaving a free end where an effort E is to be applied, the rope attached to the axle is wound round in the opposite direction and the load attached to the free end, For each complete rotation of the wheel, there is one complete rotation of the axle.

Let a and b be the radii of the wheel and axle respectively. Then

Velocity ratio=Distance moved by EffortDistance moved by Load=2πa2πb=ab

Applications of the wheel and axle are:

steering wheel of an automobile

Doorknob

windlass.

treadmill

windmill

waterwheel

GEAR:

Gear is a toothed wheel or cylinder used to transmit rotary or reciprocating motion from one part of a machine to another. Two or more gears, transmitting motion from one shaft to another, constitute a gear train.

Gear work on the wheel and axle principle. If toothed wheel A drives wheel B resulting in turning forces,

Velocity ratio=Number of teeth on the driven gearNumber of teeth on the driving gear