THE ORTHOGRAPHIC DRAWING

This is the method of drawing an object in which the shape of an object drawn makes a right angle to the plane views. The orthographic drawing means to draw at right angles. It is also known as the working drawing. In the orthographic drawing, two principal planes are used:

- Vertical plane

- Horizontal plane

When these two planes intersect one another at right angle, it will produce four (4) quadrants (4 angles).

There are three methods of pictorial drawings and pictorial drawing can show a maximum of three views of an object. The three views usually drawn in orthographic drawing are:

- Front elevation

- Plan

- End (side) elevation.

ANGLES OF PROJECTION

The angle of projection depends on which of the four quadrants the object is placed on. The two principal orthographic projections are

- The first angle projection (English projection)

- The third angle projection (American projection)

The difference between these two is the method of the arrangement of the views.

THE FIRST ANGLE PROJECTION

The first angle projection is the projection in which the plan is located directly below the front elevation (view). What is seen from the left –hand side is drawn on the right hand side. What is seen from the right-hand side is drawn on the left hand-side. Hence, the plan is located directly below the front elevation (views)

FIRST ANGLE PROJECTION

Consider the drawing below

P

L

F			                                                                                                                                 	                                                          R

FRONT VIEWS: The front views represent what is seen looking at the front of the objection of arrow (F)

PLAN VIEWS: The plain view represent what is seen when looking from the top of the object in the direction of the arrow (P)

SIDE VIEWS: The side view represent what is seen when looking at the side of the object in the direction of arrow (L)

PROPERTIES OF FIRST ANGLE PROJECTION

The following are the properties of the first angle projection.

- The plan is always projected below the front view (elevation)

- The right-hand side view is shown on left hand side of the front view

- The left-hand side is shown on the right hand side of the front views

THE THIRD ANGLE PROJECTION

This is the projection in which the plan is located directly above the front view (elevation). What is seen on the left–hand side is drawn on the left –hand side. What is seen on the right-hand side is drawn on the right side. Hence; the plan is located directly above the front elevation (views).

Considering the block diagram below

P

L

F

R

PLAN VIEWS: The plan views represent what is seen when looking from the top of the object in the direction of the arrow (P)

FRONT VIEWS: The front views represent what is seen when looking at the front of the object in the direction of the arrow (F)

SIDE VIEWS: The side views represent what is seen when looking at the side of the object in the direction of the arrow (L)

PROPERTIES OF THIRD ANGLE PROJECTION

The following are the properties of third angle projection.

- The plan is always projected above the front views

- The right hand side view is shown on the right hand side view of the elevation (view)

- The left hand side view is shown on the left hand side of the front viewing the direction of the arrow

DIMENSION TECNIQUES

- All dimensions on the working drawing must be in the same unit (millimeter).

- A working drawing must contain all essential dimensions.

- Same dimension should not be repeated in different views to avoid overcrowding of dimension lines.

- Projection and dimension lines must be thin; continuous lines and projection lines should extend beyond the dimension lines.

- Dimension lines should be at least 10mm away from the outlines, while the projection lines should start at least 1mm away from the outlines.

- Projection and dimension lines must not go across other lines in avoidable cases.

- Overall dimension should be placed outside smaller dimension.

- Arrow head must be sharp and slim. It must touch the projection line. etc.

PERSPECTIVE DRAWING

Perspective drawings are the pictorial drawing which shows the three (3) dimensions of an object at once. In a perspective drawing, all the lines on an object converge to a point called vanishing point (P).              P