TOOLS AND MACHINE

Before any wood can be used, it has to be cut from the forest and transported to sawmill where further cutting to marketable sizes is carried out. Even after sawmilling operations like; conversion processes, pieces produced from the sawing of the log will still undergo some kind of sawing to bit and pieces in the attempt to produce a work piece.

In the modern day sawing, through improved technology, various machines are now available mainly to cut wood accurately and much faster.

PORTABLE POWERED HAND TOOLS

- HAND DRILL: This is the commonly used. It takes the shape of a gun with the switch set at the trigger point. Hand drills are operated by electric motor to which the chuck’s spindle is connected and driven by it. Various sizes of drill can be connected to the drill’s chuck using the key. The main use of these tools is to bore hole.

- BELT SANDERS: The belt sanders are powered by an A.C motor which rotates one of its rollers. The two rollers are connected by a belt which has been coated with an abrasive. The belts are made of various sizes. Width and length sizes vary from 2 ½-4 (i.e. 6.3 cm-10cm) and 14-24(i.e. 35cm-60cm), respectively. To operate the belt sander, a belt of right size is fixed by slotting into the machine over the rollers. When the machine is switched on, the belt transmits the motion to the second roller and the same time rotates around the roller at a good speed. It is used for sanding the surface of wood.

- SABER SAW: this is used to cut complex shapes in wood. The wood in this case may be thicker in size than what fret saw can do. The saw is powered by an electric motor vibrator.

- MITRE SAW: It is a power saw with special quick adjustment device that can be used to cut wood at different angles. It has a saw head with tangential deviation of 45˚ with stable and precise bar guide. It contains a pedestal which supports the table and the knife. It is used for cutting mitre. It can also be used for squaring edges of timber.

- CIRCULAR SAW: This one uses saw of different sizes and teeth. It is powered also by electric motor. The saw can be adjusted to cut to depth and can be used on both timber and laminated board.

WOOD WORK MACHINE

- THE WOOD LATHE: When turning operation is to be carried out, wood lathe machine is used. There are two kinds of turning namely:

- Turning between centers and

- Turning on the face plate, both inside and outside.

The basic features of the wood lathe are:

- THE HEAD STOCK: This has a treaded spindle on which the face plate and the chucks are mounted.

- THE TOOL REST: This feature is a movable item on the bed which is capable of moving forward or backward in any position.

- TAIL STOCK: This conical center has a shank that fits into the tail stock.

- FACE PLATE: They are accessories of the wood lathe fitted to the head stock’s main spindle for inside or outside turning. The plate usually screwed to the spindle.

USES OF LATHE

The two common uses of lathe machines are:

- Cutting chisel and gauge

- Scrapping with scrapper, both within centers or on the face plate.

- THE CIRCULAR SAWING MACHINE: This derives its name from the circular saw fitted to the machine which may either be used for cross cutting or ripping (i.e. cutting along the grain of wood). It has other features apart from the circular saw such as:

- An electric motor :This is the source of mechanical energy for powering the saw

- Switch control: This is a device for putting the motor on or off.

- Fence with locking lever: This is a feature on the machine to guide the wood piece toward the saw.

- Control wheel: These controls the depths cut off and are used for tilling the saw blade.

USES

The machine is used for re-sawing timber to accurate sizes for joinery, furniture and other wood work.

- SURFACE PLANNER: This machine is designed for planning work, the most laborious wood work operation. Essential features of the machine are:

- An electric motor with switch :This powers the rotating blades

- A fence: to guide the work piece into the running blade.

- A guard: This is a safety device mounted over the cutting blades.

- The cutters: These carry a number of blades.

USES OF THE SURFACE PLANNER

- It is used mainly for planning faces and truing edges.

- It can also be adopted for rebating, beveling, chamfering and tapering

- THICKNESSING MACHINE: This machine is used after planning operation on the surface planner. Its essential features are:

- Electric motor with switch

- A feed table over which the work piece is placed

- Circular cutter blocks with two or four blades

- Anti-friction roller which are used to control the speed at which wood is fed into the cutters.

USES

The thicknesser is used to produce correct thickness by setting the cutter to the desired size. The wood passes between two cutters.

- BANDSAW MACHINE: It is a machine with a special blade called the band saw. The band saw takes the form of a rubber band with one edge toothed. It is fitted over two pulleys with the lower pulley as driver, so that the saw runs as an endless saw blade. Essential features are:

- The saw guide: which gives support to the body of the blade.

- Adjustable fence: for guiding the timber to be cut.

- Band saw compensating and track arrangement: These allow for changes in the strain on the band saw in operation.

USES

It is adopted strictly for cutting straight line cuts and circular or complex shapes and thin wood.

- DRILL PRESS: Drill press comes in various sizes and design. Its essential features are:

- The spindle: This is located and rotated in vertical position.

- An electric motor and switch control: To drive the chuck and control motor respectively

- A chuck: for holding drill bits

- Ground column: for supporting the head.

- A titling table: Upon which the work piece is placed can be adjusted with a locking clamp.

- Feed lever: This moves the spindle in it enclosure downward.

USES

It is used for drilling operation.

WOOD WORK PROJECTS

STAGES IN WOOD WORK

- DESIGN STAGE: Design is about deciding what size, shape, cost, materials and the use to which, the article produce is put.

- DRAWING STAGE: This stage is an extension of the first because what is conceived is to be drawn on paper.

- FABRICATION STAGE: The working drawing is used together with what is called the cutting list.

COMMON WOOD WORK JOINT

- BUTT JOINT

Uses

- Window frame construction

- Roof construction

- For cabinet making

- TEE BRIDLE JOINT

Uses

- For bracing

- It is used at king post and assembling of the beam

Others wood work joint are:

- Halving joint

- Dovetail joint

- Tongue and groove

- Mortise and tenon

- Housing joint

- Dowelled joint.

CLASSFICATION OF JOINTS

- Framing joint: Framing joints are used in the construction of frame.

- Widening joints: These involves joining mainly planks, edges to edges to produce a wider board.

- Angle joint: Also called box joint and is used in the formation of angles in wood construction.

NON WOOD MATERIALS

Non wood materials are material used in wooden constructions that do not possess the characteristics and nature of wood.

Adhesive –This is usually a liquid substance which has power of bonding two or more material together. Different types of adhesives are:

- PROTEIN ADHESVIES :They include

(a)Vegetable protein glue

(b) Animal protein glue

(c) Blood albumen glue

(d) Casein glue.

2. SYNTHETIC RESIN ADHESIVE: They are adhesive sourced from organic origins which react chemically to weather. There are two types

(a) Thermoplastic types: These are the types that can be soften and removed after bonding by heat e.g. P.V.A.

(b) Thermosetting type: These have a permanent bonding when glued e.g formaldehyde, resorcinol etc.

ABRASIVE

Abrasive are cloth or paper with hard mineral which produces sharp and rough cutting surface that has the ability of wearing way wood and other material by rubbing. Examples are:

- Emery

- Crushed glass

- Aluminum oxide

- Crushed quartz

- Silicon carbide

Abrasive can also be classified as

- Very fine

- Fine

- Medium

- Coarse

- Very coarse

HINGES

They are non-wood material used for installing doors, gates and windows. They are also used in installing covers to boxes, shelves etc. They are three main types of hinges

- Tee hinges

- Flat hinges

- Butt hinges

TOOLS AND CUTTING

Different kinds of cutting tools can be used to cut out different shapes. These include:

- Chisel

- Set snips

- Bench shear

- Hacksaw

Each tool is selected on the basis of two factors the thickness of the metal to be cut and the shape to be cut.

FILES AND FILLING

Filling operation on metal is the process of removing excess metal on a metal. There are different types of files used in metal work. When using files, it is necessary to know how to safely handle them.

- Never use a file without handle

- Never use a file on a welded joint

- Never use a file on steel, but slightly on the bench vice before filling

- Clean always the files.

TYPES OF FILES

- Flat file

- Hand file

- Warding file

- Square file

- Triangular file

- Round file etc

SOLDERING AND BRAZING

SOLDERING: is the process of joining two or more metals together by the use of heat. There are two types of soldering

1. HARD SOLDERING: It is a form of soldering which requires a high temperature. Hard soldering may involve brazing and silver soldering. It is used for hard/tough materials and where great strength is needed.

2. SOFT SOLDERING: It is a form of soldering which requires a low temperature. It uses soft solder, an alloy of lead, tin, antimony and bismuth. It is used for light material that does not need excess heat and also where great strength is not necessary.

FLUX

Flux is needed in both hard and soft soldering. The reasons for this include:

- To keep metal clean when heating

- To break down surface tension of the solder and allow easy flow of flux.

Tools required for soldering

- Gas heater

- Straight bit

- Hatchet bit.

BRAZING- It is a process of joining two or more metals together using a solder called spelter. An alloy of copper, zinc strip and wire where strength and high level of heat is required. The following are required for the brazing process:

- Heat source

- Filler metal

- Flux

There are three types of brazing

- Copper brazing

- Silver brazing

- Aluminum brazing